# Stabilizing Plasma as can be applied in Tokamaks

This is my attempt to implement the neural network architecture proposed in the paper Provable imitation learning for control of instability in partially-observed Vlasov–Poisson equations (arXiv.org/2605.05081v1)

The aim of this study is to find ways to keep plasma stable (i.e. hold it near a spatially uniform equilibrium) by applying an external electric field. To completely understand the purpose of this work first I must explain what plasma is.

# Plasma

Plasma refers to the fourth state of matter. The progression that occurs during heating from solid, to liquid and gas extends further than just that. Further heating results in atoms coming apart and electrons getting stripped from their nuclei. What is left is free negative electrons and positive ions moving independently.

Ionization is what makes gas electrically conductive and responsive to electromagnetic fields. Neutral gas doesn’t care about a magnetic field but plasma does.

As a result of the conductivity, plasma has a tendency to behave strangely. As opposed to gases where particles only interact when they physically collide, in plasma every charged particle feels the electric field of every other charged particle at a distance. Interactions are long-range and collective. This is what makes plasma physics complicated since particles create an electric field and the field steers the particles in a closed feedback loop. These tiny perturbations result in instabilities that grow spontaneously.

It’s no secret that the main interest behind plasma is its application in generating energy. During fusion, hydrogen plasma is heated to ~100 million degrees and held together long enough such that nuclei fuse and release enormous energy, without the radioactive waste of fission. Since no material container can touch matter that hot, it is confined magnetically in a doughnut-shaped device known as a tokamak .

# Our Aim

As stated in the introduction and reiterated in the ‘Plasma’ section, our aim is to identify the perturbations caused by the electromagnetic field in plasma environments and stabilize them.

If we could see the entire phase-space distribution f(x,v) stabilizing becomes almost trivial since all we would need to do is cancel the plasma’s own internal electric field H = -E, which would collapse the dynamics to pure free streaming. Free streaming then stabilizes via Landau damping / phase mixing.

This ‘cancel-the-field’ controller is what is later referred to in this study as the expert.

In a real device however, we can’t actually see f(x,v) . We see a handful of noisy density readings at a few sensor locations and density doesn’t reveal the velocity-space structure that actually governs stability (the Penrose condition). So the expert is not implementable.

To that point the authors proposed imitation learning whereby we train a neural network to map a short history of these sparse density readings to the expert’s control action, then deploy it in a closed loop. The authors proved that the learned controller inherits exponential stabilization up to an error floor. That floor is characterized in terms of how rich the observations are and how ‘complex’ the initial distribution is (via a new entropy notion) and validate it on a 1D1V two-stream instability with only 4 sensors.

# The Physics and Control Problem

The system is the 1D-space, 1D velocity Vlasov-Poisson equation on a periodic domain:

- Transport: ∂tf+v∂xf-(Ht(x)+Et(x))∂Vf=0

- Poisson: ∂xxvt=1-ρt(x), with ρt(x)=fdv & Et=∂xvt

- Ht(x)  is the external control field.

Et is the plasma’s self-consistent internal field.

Any spatially uniform f=μ(v) is a stationary solution, but generically an unstable one since perturbations will grow. The control objective is to design Ht observations so the system stays near that equilibrium (equivalently the electric energy stays small) for as long as possible.

The 1D1V reduction isn’t just for convenience either. Under a strong enough uniform magnetic field (B~1ε), fast perpendicular gyration averages out and under slab symmetry the full 3D3V system reduces at leading order to exactly this 1D1V model along the field line. So the reduced model still carries the velocity space structure that drives kinetic instabilities.

The obstacle faced here is partial observability.  As stated earlier stability of a kinetic plasma depends sensitively on fine velocity-space features (formalized by the Penrose condition).

But two very different distributions can produce nearly identical macroscopic density profiles. Real feedback systems (DIII-D) only see sparse macroscopic diagnostics therefore the information we can condition on is provably lossy w.r.t the thing to be controlled.

This is the whole tension here, and why a single-shot reconstruction can’t work so we need to exploit history.

The expert (privileged controller) and why it works:

Full state control is exact field cancellation: H*=-Et. If we plug that in and the transport equation becomes free-streaming:

∂tf+v∂xf=0

Under regularity/velocity-decay assumptions on the initial condition (Assumption 1 - Smooth , factorially-bounded derivatives, polynomial velocity decay; a density * mixture-of-Gaussians satisfies it), the free stream electric energy decays exponentially,

||E||2L2 ≤c1e-c2t .

This is the Landau-damping / phase mixing mechanism (Mouhot Villiani).

Since a real partially-observed controller needs some history before it can act, the actual expert policy is: run uncontrolled for a short burn-in [0,t0], then switch to H=-Et. Proposition 1 shows this still gives exponential energy decay (the phase mixing effect is robust to that initial transient. This expert is exactly baseline B2 in the experiments of the original paper.

The imitation learning algorithm:

Now, distill that unimplementable expert into something that only sees sparse density history:

- Sample i.i.d (identifiable, identically distributed) initial conditions from v0; simulate each under the expert policy to get trajectories.

- Along each trajectory, collect the noisy density at N sensors at cadence η, forming an observation window D[t-to,t] (an N*(t0η+1) matrix)

- Learn a time-homogenous policy π that maps that window to the expert action -Et by empirical risk minimization (least squares on the L2 control error, pooled over time steps. This is textbook behaviour cloning.

- Deploy online: Ht=π(D[t-t0,t]) for t>= t0 , with the window collected live.

What this study actually proves:

Proposition 1:

The expert (burn-in field cancellation) stabilizes exponentially.

Theorem 1:

The learned policy inherits exponential stabilization, with an error floor

≈c3(t)⋅[infxR(π)+(logN(ε,)n+ε]

That’s approximation error (can the policy class  match the expert from partial observations at all?) plus a standard converting number / n statistical term.

The prefactor c3(t) can grow with time (Gronwall + behavior cloning covariate shift), so this is a finite horizon guarantee that small approximations buys you a long stable window and whether the exponential blow up is avoidable is left open. This is why empirically, everything eventually diverges.

Theorem 2:

Bounds the minimal risk infπR(π) by three interpretable pieces:

- An unavoidable discretization floor, c(η2+e-c0N), from a finite sampling rate.

- A bias-variance trade-off in history length k-averageing more past observations cuts variance σ2k but adds bias (kη)2p because old data is stale.

- A statistical term featuring a new ε-resolution entropy Hε(v0)=-logv0(B2(f,ε))dv0(f) that measures the complexity of the initial distribution at scale ε . Low entropy→small cloning loss→good stabilization, and crucially the policy class adapts automatically without knowing v0 which is the selling point over Bayesian-inverse problem approaches that need the prior exactly.

# Vlasov-Poisson Equations

A VP system is a way to describe Plasma not as a  swarm of individual particles but as a density in phase-space.

Density:

A function f(t, x, v) telling us how many particles sit near position x with velocity near v at time t.

To understand VP model, focus on two statements:

- Particles stream and get pushed around by an electric field (Vlasov)

- The field is generated by the particle’s own charge (Poisson)

The Vlasov Equation

Written in 1D1V convention:

∂tf+v∂xf-(Ht+Et)∂vf=0

This is a conservation law:

- f is constant along the trajectory of each particle.

- The 3 terms are just the chain rule for df/dt along Newton’s equations of motion:

∂tf: How f changes in time at a fixed point

v∂xf: Particles carry their density with them as they move in space at velocity v. This is the free streaming i.e fast particles drift right, slow ones lag.

- (Ht+Et)∂vf: The force reshuffles particles in velocity. The acceleration here is the total field, the plasma’s self-field Et plus your external control Ht.

Since the RHS has a zero (no collision term), this is a collisionless model (particles never scatter off each other directly). They only feel each other through the shared mean field.

Since f is conserved along trajectories while phase-space volume is preserved (Louisville’s theorem), f can’t pile up or compress. It can only be stirred (stretched into ever finer filaments in velocity). That filamentation is the mechanism behind Landau damping and behind why the expert controller works.

The Poisson Equation

The field is self-generated. The field Et is made by the particles.

We integrate f over velocity to get the spatial charge density, ρt(x)=fdv then solve:

∂xxvt=1-ρt,Et=∂xvt

The ‘1’ is a fixed neutralizing background (think heavy ions); ‘1-ρ’ is the net charge. When the mobile particles are spread perfectly uniformly (ρ=1) everywhere the net charge vanishes, there’s no field and you sit at equilibrium. Any bunching creates a field that pushes back, or for the wrong velocity distributions, pushes outwards and grows.

# The Governing Equations

- The Plasma Dynamics being controlled

The core governing system - The 1D1V Vlasov-Poisson equations on [0,T]**ℝ

∂tft(x,v)+v∂xft(x,v)-(Ht(x)+Et(x))+Et(x))∂vft(x,v)=0

∂xxvt(x)=1-ℝft(x,v)dv

Et(x)=∂xvt(x)

The first is the Vlasov transport equation (f conserved along Newtonian trajectories, driven by the total field Ht+Et;

The second is the Poisson closing the field self-consistently from the charge.

The third defines the field from the potential. Any spatial homogenous ft=μ(v) is a stationary solution, but generically an unstable one, which is the whole point of controlling it.

The reduced model’s parent i.e the full 3D3V system:

δtf+v⋅∇xf+(E+v*B)⋅∇vf=0

This is the equation whose phase mixing gives the stabilization

- The Observation Model

The macroscopic density (the only observable quantity’s raw form)

ρt(x)=ℝft(x,v)dv

Noisy sparse sensor readings at N uniform locations xi=iN and times kη:

pi,k =ρkη(xi)+Wi,k,               Wi,k~𝒩

The pooled observation window fed to the policy, an N*(t2η-t1η+1) matrix

pi,k=ρkη(xi)+Wi,k,                  Wi,k~𝒩(0,σ2)i.i.d

The controller always uses the trailing window D[t-t0,t]

- The Three Control Policies

Full Information (privileged) optimal control - exact field cancellation:

H*t(x)=-Et(x)

The expert policy that the network imitates. This is the uncontrolled burn-in, then cancellation:

Htexpert(x)=0                             for  t∈[0,t0]

=-Et(x)               for t>t0

The deployed learned policy which is the network acting on the live observation window.

Htπ(x)=0                for t<t0

=π(D[t-t0,t])(x)      for t>=t0

- The Imitation-Learning Objective

Random Initial conditions: f01, ⋅⋅⋅ ,f0n~v0   i.i.d

Empirical risk minimization / behavior cloning

π=argminπ∈Rπ

Rπ=(1n)jnℓ (j)(π)

ℓ(j)(π):=ηT-t0t0<=kη<=T||π(D(j)[kη-t0,kη])+E(j)kη||2𝕃2(T)

The target is -E, so the loss drives π (observation) toward -E. I.e toward the expert action.

Its population (continuous time) counterpart (Eq. 9 in the original paper), whose infimum over the class is the irreducible approximation error.

ℓ(π;z):=1T-t0t0TE[π(Dz(j)[t-t0,t]+Et(j)2𝕃2(T)]dt

ℝ(π)=𝔼z[ℓ(π;z)], approximation error = infπ∈R(π)

- The Functionals and Inequalities in the Guarantees.

The analyticity norms that state the regularity hypothesis

(Assumption 1: f0εq(λ0)<=B0

fcqm=supα=msup(x,v)(1+v)q∂αf(x,v)

fEq(λ)=m=0∞λmm!fcmq

The expert stability guarantee: exponential decay of electric energy

||Et||2𝕃2(T)<=c1e-c2t∀  t>=0

The learned policy guarantee (Theorem 1), where N(ε,,⋅∞) is the ϵ-covering number of the policy class:

Etπ2𝕃2(T)<=c1e-c2t+c3(t)⋅{infπ∈R(π)+logN(ϵ,,⋅∞n+ϵ

where the constants c1,c2,>0 are the same in position 1.

The ε-resolution entropy – the scale sensitive complexity measure for the initial distribution.

Hε(v0):=-logv0(𝔹𝕃2(f,ε))dv0(f)                 ∀   ε>0

The approximation error decomposition (Theorem 2) valid for any p<=q-2:

infπ∈R(π)<=c(η2+e-c0N)+c infk∈ℕ+,ε>0{(kη)2p+ε2+σ2k⋅Hε(v0)+log(Tε)N}

where c,c0>0 are constants depending on the regularity parameters in Assumption 1 and the integer constant p.

The three groups are the unavoidable discretization floor (finite sensors N, finite cadence η), the staleness bias from using a length-k history, and the variance term that carries the entropy–longer history k cuts variance σ2k but raises the (kη)2p bias.

- The experiment’s concrete instantiations

Two-stream Maxwellian equilibrium (Eq. 11), v=2.4:

μ(v;v)=122πexp(-v-v2)+122πexp(-(v+v)22)

Where we fix the stream separation parameter at v=2.4.

To challenge the control policy, we construct the initial state using a dual-mode perturbation:

f0(x,v)=μ(v;v)(1+εη(x))                        ε=0.05

where spatial perturbation η(x) superposes a macroscopic low-frequency mode and a microscopic high-frequency mode:

η(x)=cosk12πxLx+ϕ1,ϕ2~Unif(0,2π) are randomized for each trajectory to evaluate the robustness of the policy

Observation tensor (Eqn 14), N=4, K=50:

Dj=(ρi,j-ℓ)        ∈ℝN*K         1<=i<=N ,   0<=ℓ<=K-1

Scale-aware hybrid loss (Equations 15 and 16), Mf=8, so 2Mf=16. Coefficients per step, with a geometric per-mode weight.

L=α𝓛abs+β𝓛rel

α and β are balancing hyperparameters.

Let ck,j denote the ground truth Fourier coefficients for mode k at the jth step within the prediction window and let ck,j denote the corresponding network predictions. The absolute and relative error terms are defined as:

𝓛abs=1Kj=1Kk=12Mfwck,j-ck,j2

𝓛rel=1Kj=1Kk=12Mfwkck,j-ck,jck,j2

where wk∝k-1 is a static decay-weight applied to the kth Fourier mode.

Baseline controllers:

B0: Ht(x)≡0

B1: Ht(x)=-E't(x)

Poisson: zero-pad to nx=1024 → inverse FFT

B2: Ht(x)=-Etruet(x)⋅1t>=t0

Evaluation metric - macroscopic electric energy

E(t)=120LXEt(x)2dx

# Understanding the Data

The data used for this study is completely synthetic and is regulated by the governing equations in the previous sections. The data in question is the Vlasov-Poisson Simulator.

The dataset is a synthetic collection of (what the sensors would see and what the ideal controller would do) pairs, manufactured by running the physics forward and watching the expert.

Two views of trajectory:

- Each simulated trajectory is recorded in two parallel ‘views’, and the whole method lives in the gap between them.

- The privileged view is the full-phase distribution ft(x,v) on the 1024x1024 grid, and everything you can compute exactly from it–the density ρt, the potential vt, and the internal electric field Et.

This is what an omniscient observer sees. It’s used for three things:

To run the simulation

To compute the training labels

To produce the phase-space snapshots in the results

It is never available to the deployed controller.

- The observed view is what a real diagnostic would give you: the density sampled at just 4 fixed sensor locations, at discrete times, corrupted by Gaussian noise.

This is the only thing the network is actually allowed to consume. The entire premise – controlling a kinetic instability from macroscopic measurements – is the statement that you must act using the observed view while the physics is governed by the privileged view.

The data therefore is imitation-learning data: input = a short history of the observed view; label = the expert’s action, which is derived from the privileged view.

How data is prepared

For each trajectory, the pipeline is:

- Draw an initial condition f0~v0

V0 is an extremely narrow family – the two stream Maxwellian μ(v;v) with v=2.4, perturbed by f0=μ(v)(1+εη(x)) with ε=0.05 and η as sum of a mode=1 and a mode =-5 cosine. Everything is fixed except two phase shifts  ϕ1,ϕ2~Unif(0,2π) which are redrawn for every trajectory.

- Simulate the expert trajectory

With the semi-Langrarian solver on the 1024x1024 (x,v) grid, at Δt=0.02, to T=80. On the burn-in [0,t0=1] no control is applied, so the full self-consistent Vlasov-Poisson runs. For t>t0 the expert applies H=-Et, which cancels the field and reduces the evolution to pure free-streaming. This is why authors describe the training data as coming from ‘unforced free streaming evolution’ – after burn-in, the field isn’t driving the particles at all; f just phase-mixes ballistically, and as it does ρt relaxes toward uniform and Et decays by Landau damping. A consequence you must plan for; the labels span many orders of magnitude, huge early but tiny later.

- Record at each timestep the two views

The observed view: subsample ρt to the 4 sensor locations, add noise Wi~𝒩(0,σ2). The label: solve Poisson for the exact Et and store the target control field -Et, represented as its first 2Mf=16 truncated Fourier coefficients. Note that the field solve after burn-in is done only to produce the label – free streaming doesn’t need it.

The result of one trajectory is a long time series: at each control time tj (from t0 to T), one input window paired with one target coefficient vector.

Structure of a single training example

The input at control step j is the observation tensor (Eqn 14)

Dj=(ρi,jℓ)      for 1<=i<=N=4 ,      0<=ℓ<=K-1=49  → SHAPE 4x50

i.e 4 sensors by the trailing 50 time steps of the noisy density. Before it enters the network, it’s augmented with first order temporal differences Δρ (consecutive-step differences, giving the network an explicit velocity-of-density channel alongside the raw values – effectively doubling the input features.

One detail worth internalizing about the window: K=50 and Δt=0.02 give K⋅Δt=1=t0 exactly. The observation cadence η equals the solver step Δt, and each window spans precisely one burn-in horizon of history. This is the amount of past the deployed policy is defined to use D[t-t0 , t]

The label at step j is the target control field -Etj(x), stored as the 16 real Fourier coefficients ck,j(Mf=8 modes → 8 complexes → 16 real numbers). The network predicts ck,j and inverse FFT turns them back into the spatial field. Working in this truncated spectral space is both the label representation and the output representation.

Stacking over all control steps and all trajectories gives the full dataset. Per trajectory there are roughly (T-t0)Δt≈3950 such (window, coefficients) pairs, so even a modest number of trajectories yields a large pool of examples – though you should note the windows within one trajectory heavily overlap.

# The Neural Network

## Causal Temporal Convolution Network

The TCN is the part where the physics-to-tensor mapping is the least obvious.

It converts 8 raw sensor channels into 64 learned temporal features, at every one of the 50 timesteps, using only past information.

Shape-wise (B, 8, 50)  → (B,64, 50) . The time axis is deliberately preserved – The TCN doesn’t summarize the window, it enriches each timestep with context from its past, and only later does attention integrate globally and mean-pool collapse it.

The 5 blocks.

| Block | Dilation | Pad/Chomp | Shape in → out | params | Cumulative RF |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | (8,50) → (64,50) | 14,656 | 5 |
| 2 | 2 | 4 | (64,50) → (64,50) | 24,832 | 13 |
| 3 | 4 | 8 | (64,50) → (64,50) | 24,832 | 29 |
| 4 | 8 | 16 | (64,50) → (64,50) | 24,832 | 61 |
| 5 | 16 | 32 | (64,50) → (64,50) | 24,832 | 125 |

In total the TCN block accounts for 113, 984 params (~59% of the whole model’s 192k params) The time axis is 50 throughout (the blocks preserve it) Channel count changes only in block 1.

The cumulative receptive field is the whole reason for the dilation schedule. Each block has two convs fo kernel 3, so it adds 2⋅(k-1)⋅d=4d to the RF. Doubling d makes the R grow geometrically – 5, 13, 29, 61, 125 – reaching 125 >= K=50 within only 5 blocks. Stacking non-dilated convs to cover 50 steps would need ~25 blocks and roughly five times the parameters.

Layer by Layer view of each TCN block:

| input | (1, 64, 50) |
| --- | --- |
| [0] Conv1d (WN, k=3, d=4) | (1, 64, 58) ← left-pad 8 → length grows |
| [1] Chomp 8 | (1, 64, 50) ← drop last 8 → causality enforced |
| [2] GELU | (1, 64, 50) |
| [3] Dropout (0,1) | (1, 64, 50) |
| [4] Conv1d(WN, k=3, d=4) | (1, 64, 58) |
| [5] Chomp (8) | (1, 64, 50) |
| [6] GELU | (1, 64, 50) |
| [7] Dropout (0.1) | (1, 64, 50) |
| residual | (1, 64, 50) ← identity (1x1 conv only in block 1) |
| GELU (out+residual) | (1, 64, 50) |

Conv1d (dilated)

A 1d convolution along time, shared across all 50 positions. With k=3, d=4 each output at time step t is a learned mix of the input at steps t, t-4, t-8 – the dilation skips intermediate steps so the filter spans 9 steps while holding only 3 taps. It’s a full mix across the 64 input channels, so it learns cross sensor relationships too, not just per-sensor patterns. Weight normalization reparameterizes each filter as magnitude x direction, which decouples scale from orientation and stabilizes optimization – relevant here because targets span orders of magnitude.

The padding / chomp pair is the causality mechanism, and it’s the subtlest part. PyTorch’s Conv1d pads asymmetrically, which would let step t peek at t+1. The Bai-style fix: pad (k-1)⋅d=8zeros on the left only, which makes the output length grow 50→58 then chomp (8) discards the last 8 positions returning to 50. What survives is an output where step t is a function of inputs <= t and nothing later.

GELU

Smooth non-linearity, element-wide shape unchanged. Without it, the stack collapses to a single linear filter, unable to represent the amplitude / phase relationships that encode the hidden velocity -space state

Dropout(0.1)

Training only channel noise; identity at evaluation. Regularizes against overfitting the heavily-overlapping windows. Consecutive training examples differ by one timestep, so effective sample size is far below the raw example count.

Residual connection

Output = GELU(conv_path + input). Gradients reach early blocks directly and each block only has to learn a correction to what came before. The dow projection is a 1x1 Conv1d in block 1 (to reconcile 8 channels with 64) and identity in blocks 2-5.

## Attention Mechanism

TCN hands over (B, 64, 50)

A transpose makes the 50 timesteps into tokens: (B, 50, 64) - 50 tokens, each a 64-dim feature vector describing ‘what was happening around this timestep’. The encoder maps (B, 50, 64) → (B, 50, 64). Shape unchanged – it’s a sequence-to-sequence refinement, not a summarizer. Collapsing happens later at mean pool.

Parameter inventory (one encoder layer = 33,472; 2 layer = 66,944 params)

| Parameter | Shape | Count |
| --- | --- | --- |
| self_attn.in_proj_weight | (192, 64) | 12, 288 |
| self_attn.in_proj_bias | (192, ) | 192 |
| self_attn.out_proj.weight | (64, 64) | 4096 |
| self_attn.out_proj.bias | (64, ) | 64 |
| Linear1.weight (FF up) | (128, 64) | 8192 |
| linear1.bias | (128, 64) | 128 |
| linear2.weight (FF down) | (64, 128) | 8192 |
| linear2.bias | (64, ) | 64 |
| norm1.weight / bias | (64, ) x 2 | 128 |
| norm2.weight / bias | (64, ) x 2 | 128 |

PyTorch stacks Wq, Wk , Wv into a single matrix (3x64 = 192 rows) for one fused matmul. If we rebuild with three separate Linear (64, 64) layers, we get identical math and identical parameter count – just a different state-dict layout.

Layer by Layer

0. Positional Encoding (before the encoder, 0 params)

Attention is permutation-invariant – shuffle the 50+ tokens and you get the same set of outputs, reordered. It has no idea what earlier means. Sinusoidal encodings are added to each token so position is baked into the features.

- QKV Projection x@WqkvT+b : (1, 50, 64) → (1, 50, 192)

Split into Q, K, V of  (1,50,64) each. Each token emits three different views of itself – a query (what am I looking for?) a key (what do I offer to others?), and a value (what content do I pass along if selected?)

- Reshape into heads (1,50,64) → (1, 4, 50, 16)

The 64 dims are split into 4 heads of head_dim=16. Each head does the whole attention operation independently on its own 16-dim subspace, so different heads can specialize – one might track slow oscillation, another a fast one, another cross-sensor lag. The heads are then concatenated back, so multi-head costs nothing extra in parameters versus single head; it just partitions the same budget.

- Attention scores : scores = QKTd=16→ (1,4, 50, 50)

This is the heart of attention. A 50 x 50 matrix per head, where entry (i, j) is the affinity between token i’s query and token j’s key. The 16 divisor keeps the dot query from growing with dimension and pushing softmax into saturation, where gradients vanish.

- Softmax A= softmax (scores)

Rows sum to 1. Each token now has a probability distribution over all 50 timesteps – a set of weights saying how much to draw from each other timestep.

- Context = A@V → (1, 4, 50, 16)

Each token’s output is the attention-weighted average of all tokens’ value. This is the actual information movement – a token at step 49 can pull content directly from step 0 in a single hop.

- Concat + output projection

Heads recombine (1, 4, 50, 16) → (1, 50, 64), then W0 (64, 64) mixes across the head boundaries so heads can interact rather than staying isolated subspaces.

- Add + LayerNorm (post-norm) x = LayerNorm(x + Dropout(attn))

The residual preserves TCN features (attention edits rather than replaces) and LayerNorm normalizes each token each token’s 64 dim vector to zero mean / unit variance with learned scale and shift, keeping activations well conditioned across layers.

- FeedForward Linear (64 → 128) → GELU → Dropout → Linear (128, 64)

Applied independently to every token (no mixing across time). Attention moves information between token (no mixing across time) Attention moves information between tokens ; the FF network then processes what each token gathered. The 2x widening gives it room for a non-linear transform before projecting back.

- Add & LayerNorm again